UNIVERSITY OF CALIFORNIA

Los Angeles

Shear Wave Velocity as Function of SPT Penetration Resistance and Vertical Effective Stress at California Bridge Sites

A thesis submitted in partial satisfaction

of the requirements for the degree Master of Science

in Civil and Environmental Engineering

by

Naresh Bellana

2009

The thesis of Naresh Bellana is approved

_________________________________

Jonathan P. Stewart

_________________________________

Mladen Vucetic

_________________________________

Scott J. Brandenberg, Committee Chair

University of California, Los Angeles

2009

DEDICATION

To

my parents,

family

and  friends

# Table of Contents

Table of Contents	iv

List of Figures	v

List of Tables	vii

Acknowledgments	viii

ABSTRACT OF THE THESIS	ix

1.	Introduction	1

2.	Literature Review	4

2.1.	Influence of Overburden Stress	7

3.	Development of Data Set	10

4.	Statistical Regression	18

4.1.	Regression Analysis	18

4.2.	Residuals	22

4.3.	Influence of Surface Geology	24

4.4.	Effect of Omitting v’ Term	26

4.5.	Comparison with Published Relations	28

5.	Conclusions	30

6.	Appendix	33

7.	References	62

# List of Figures

Figure 1. Modulus reduction curves for (a) sand (Seed and Idriss 1970) and (b) clay (Vucetic and Dobry 1991).	2

Figure 2. Influence of overburden on Vs values associated with constant N60 but different (N1)60	8

Figure 3. Map of sites where boring logs and suspension logs were measured for this study.	11

Figure 4. Distribution of data points by soil type.	13

Figure 5. Example of data processing methodology to obtain weighted average  value for statistical regression.	16

Figure 6. Vs versus N60 and Pa/v’ for (a) sand, (b) silt, and (c) clay, and lines showing results of regression equations for various N60 and Pa/v’ values.	21

Figure 7. Residuals, defined as natural log of measured Vs values minus natural log of predicted Vs values, versus N60 and Pa/v’ for (a) sand, (b) silt, and (c) clay.	23

Figure 8. Quantile-quantile plots indicating the extent to which the error term is normally distributed for (a) sand, (b) silt, and (c) clay.	24

Figure 9. Vs versus N60 with different symbols indicating different surface geologic epoch for (a) sand, (b) silt, and (c) clay.	25

Figure 10: Residuals versus N60 and Pa/σv' for regression for all soils without overburden term.	27

Figure 11: Existing correlations from literature over the current data set	29

Figure 12. (a) Bridge no. 10-0298, boring no. 98-4 (abut.4), (b) Bridge no. 10-0298, boring no. 96-2, (c) Bridge no. 10-0298, boring no. 96-3.	34

Figure 13. (a) Bridge no. 10-0298, boring no. 98-10 (abut 1) (b) Bridge no. 28-0253R, boring no. 94B1R (Pier 9) (c) Bridge no. 28-0153R, boring no. 96-5 (Pier 8)	35

Figure 14. (a) Bridge no. 28-1053R, boring no. 95B13R (Pier 7) (b) Bridge no. 28-1053R, boring no. 96-4 (Pier 7) (c)  Bridge no. 28-1053R, boring no. 95-12 (Pier 5)	36

Figure 15. (a) Bridge no. 28-0352L, boring no.  96B-29 (b)  Bridge no. 28-0352L, boring no. 95-2 (Pier 3) (c)  Bridge no. 28-0352L, boring no. 95-1 (Pier 4)	37

Figure 16. (a) Bridge no. 28-0352L, boring no. 96B-37 (b) Bridge no. 28-0100, boring no. 96-2 (Piers 10 and 11) (c) Bridge no. 28-0100, boring no. 96-5 (Piers 31/32)	38

Figure 17. (a) Bridge no. 28-0100, boring no.  96-7 (Pier 8) (b) Bridge no. 28-0100, 95-7 (Pier 21) (c) Bridge no. 28-0100, boring no. 95B4R (Pier 25)	39

Figure 18. (a) Bridge no. 28-0100, boring no.  95B5R (Pier 35) (b) Bridge no. 28-0100, boring no. 95B2R (Pier 32/33) (c) Bridge no. 28-0100, boring no. 95B3R/95B9R (Pier 34)	40

Figure 19. (a) Bridge no. 28-0100, boring no. 95-10 (Pier 47) (b)  Bridge no. 28-0100, boring no. 95-11 (Pier 48) (c)  Bridge no. 28-0100, boring no. 95B1R (Pier 58)	41

Figure 20. (a) Bridge no. 33-0025, boring no. B6 (Pier E19) (b) Bridge no. 33-0025, boring no. B-7 (Pier E10) (c) Bridge no. 34-0003, boring no. 95-14 (Pier W6)	42

Figure 21. (a) Bridge no. 34-0003, boring no. 95-12 (Pier W4) (b) Bridge no. 34-0003, boring no. 95-11 (Pier W3) (c) Bridge no. 34-0003, boring no. 95-10 (Pier W2)	43

Figure 22. (a) Bridge no. 34-0003, boring no. 95-5 (Pier A) (b) Bridge no. 34-0003, boring no. 95-4 (c) Bridge no. 34-0003, boring no. 95-6	44

Figure 23. (a) Bridge no. 34-0004, boring no. B95-2 (b) Bridge no. 34-0004, boring no. B95-3 (c) Bridge no. 34-0077, boring no. 01-B2	45

Figure 24. (a) Bridge no. 34-0077, boring no. 01-05 (b) Bridge no. 34-0077, boring no. 01-08 (c) Bridge no. 34-0077, boring no. 01-11	46

Figure 25. (a) Bridge no. 37-0853, boring no.98-1 (Pier 4) (b) Bridge no. 38-0583, boring no. 98-4 (Bent 7) (c) Bridge no. 49-0014L, boring no.98-1 (Abut 1)	47

Figure 26. (a) Bridge no. 51-0139, boring no. 98-1 (Abut 1) (b) Bridge no. 52-0443, boring no. 99-1 (c) Bridge no. 53-1471, boring no. 95B5R	48

Figure 27. (a) Bridge no. 53-1471, boring no. 95B4R (b) Bridge no. 53-1471, boring no. 95B1R (c) Bridge no. 53-1471, boring no. 95B2R	49

Figure 28. (a) Bridge no. 53-1471, boring no. 95B3R (b) Bridge no. 53-2272, boring no. B-1 (c) Bridge no. 53-2790R, boring no. B-6	50

Figure 29. (a) Bridge no. 53-2794R, boring no. B-1 (b) Bridge no. 53-2795F, boring no. 94-21 (c) Bridge no. 53-2796F, boring no. 94-30	51

Figure 30. (a) Bridge no. 54-1110R, boring no. 98-1 (Abut 1) (b) Bridge no. 54-1110R, boring no. 98-6 (Abut 8) (c) Bridge no. 57-0857, boring no. 96-52 (Bents R48 and 49)	52

Figure 31. (a) Bridge no. 57-0857, boring no. 96-17 (Abut S48) (b) Bridge no. 57-0857, boring no. 95-2 (Pier 33) (c) Bridge no. 57-0857, boring no. 96-16 (Bent 41F,R)	53

Figure 32. (a) Bridge no. 57-0857, boring no. 96-29 (b) Bridge no. 57-0857, boring no. 96-53R (c) Bridge no. 57-0857, boring no. 96-66	54

Figure 33. (a) Bridge no. 57-0857, boring no. 96-65 (b) Bridge no. 57-0857, boring no. 96-35 (Toll Plaza North West) (c) Bridge no. 57-0857, boring no. 96-34 (Toll Plaza South East)	55

Figure 34. (a) Bridge no. 57-0857, boring no. 96-21 (b) Bridge no. 57-0857, boring no. 96-28 (c) Bridge no. 57-0857, boring no. 96-60	56

Figure 35. (a) Bridge no. 57-0857, boring no. 96-68R (b) Bridge no. 57-0857, boring no. 96-56 (c) Bridge no. 57-0857, boring no. 96-67	57

Figure 36. (a) Bridge no. 57-0857, boring no. 96-54 (b) Bridge no. 57-0857, boring no. 96-55 (c) Bridge no. 57-0857, boring no. 96-59	58

Figure 37. (a) Bridge no. 57-0857, boring no. 96-58 (b) Bridge no. 57-0857, boring no. 96-57 (c) Bridge no. 57-0857, boring no. 96-64	59

Figure 38. (a) Bridge no. 58-0335RL, boring no. B5-01	60

# List of Tables

Table 1. Some existing correlations between Vs and SPT-N	6

Table 2. Bridge name, number, location, number of borings, and surface geology epoch for sites included in this study.	10

Table 3. Rod length correction factors  (Source: Youd et al., 2001).	14

Table 4. Unit weights.	15

Table 5. Regression constants and standard deviation of error term.	19

# Acknowledgments

This research was funded by the Pacific Earthquake Engineering Research Center under contract number 65A0215.  I would like to thank Tom Shantz for providing technical guidance on this work and for providing the boring logs and suspension log data.  I would like to thank Barr Levy for helping me extract the electronic database from the boring log tif image files.

# ABSTRACT OF THE THESIS

Shear Wave Velocity as Function of SPT Penetration Resistance and Vertical Effective Stress at California Bridge Sites

by

Naresh Bellana

Master of Science in Civil and Environmental Engineering

University of California, Los Angeles, 2009

Assistant Professor Scott J. Brandenberg, Chair

Equations representing shear wave velocity (Vs) as a function of energy-corrected SPT blow count (N60) and vertical effective stress (v’) are generated by statistical regression of site investigation data at bridge sites owned by the California Department of Transportation.  The N60 values and Vs values were obtained from the same boreholes between 1993 and 2001.  The Vs values were obtained by the suspension logging method.  A total of 21 bridges and 79 borings provided 918 pairs of N60 and Vs values for regression.  Surface geology at each site was obtained from a yet-unpublished Caltrans study.  The influence of v’ has not been considered by the vast majority of published relations between Vs and N60, yet it was shown in this thesis to be a significant factor.  Significant bias may exist in previous relations that neglected the influence of v’.  Residuals, defined as the natural log of measured minus the natural log of predicted values of Vs, were found to be normally distributed and the standard deviations were quantified as part of the statistical equations.  Uncertainty in the relations is substantial, and the correlations should only be used as a rough estimate of Vs to prioritize site investigation resources by identifying whether directly measuring Vs would be worthwhile.  Geophysical measurements should be made when uncertainty in the estimated values significantly affects design or retrofit decisions.

# Introduction

A key property required to evaluate dynamic response of soil is small-strain shear modulus, Gmax. Values of Gmax are typically determined indirectly by measuring the shear wave velocity, Vs, and the mass density of the soil, ρ, and computing Gmax = ρVs2.  Small-strain shear modulus, Gmax may be determined using “undisturbed” soil samples provided that accurate stress-strain behavior can be measured at very small shear strains (i.e. less than 0.001 percent), and sample disturbance is negligible.  Disturbance is known to affect small-strain stiffness more than shear strength because weak inter-particle bonds broken during sampling contribute significantly to small-strain stiffness, hence Vs is most often measured in-situ rather than in the laboratory.

Gmax is often used in combination with modulus reduction (G/Gmax-γ) and damping (D-γ) curves to solve dynamic problems when shear strains drive the soil beyond its elastic range.  Modulus-reduction curves describe the reduction of secant modulus with increase in cyclic shear strain, γc. Damping curves describe the hysteretic energy dissipated by the soil with increase in c.  These curves can be obtained through laboratory cyclic loading tests, but are typically assumed for a given soil type. The curves for obtained by Seed and Idriss (1970) for sand and by Vucetic and Dobry (1991) for clay are shown in Fig 1.

(a)

(b)

Figure 1. Modulus reduction curves for (a) sand (Seed and Idriss 1970) and (b) clay (Vucetic and Dobry 1991).

The importance of Gmax has been widely recognized in ground motion prediction equations by implementation of site factors that modify ground motion based on the difference between a site Vs parameter and a reference Vs parameter.  For example, Choi and Stewart (2005) utilized the average Vs in the upper 30m, called Vs30, and the Next Generation Attenuation relations include Vs30 as an input parameter.

Geophysical measurements have become very common for geotechnical projects where vibrations are expected, though this was not always the case. Many older site investigations gathered information only the geologic setting, stratigraphy, and penetration resistance (SPT blow counts, or cone penetration resistance). Lack of geophysical measurements from older site investigations is particularly pertinent for bridges since many State Departments of Transportation own thousands of bridges, many of which were constructed before geophysical measurements were common.  For example, Caltrans owns about 13,000 bridges, most of which were constructed before 1970.  As ground motion prediction equations have advanced to include Vs values as inputs, there is a need to estimate Vs at the older bridges based on available information to guide retrofit evaluations.  Correlations between shear wave velocity and blow count, geologic setting, and site stratigraphy are therefore potentially useful at least as a screening tool for identifying a subset of bridges where geophysical measurements would be the most beneficial.

# Literature Review

Numerous relations between SPT blow count, N, and shear wave velocity, Vs, exist in the literature.  Early efforts utilized laboratory results to develop correlations, and the correlations were subsequently refined as field measurement of Vs became more common and data became available.  The early correlations based on field data often involved blow counts that were not corrected for energy, rod length, or sampler inside diameter.  Hence, it is impossible to know whether bias is introduced by hammer efficiency, non-standard samplers, etc.  Furthermore, various methods of measuring Vs were utilized in the correlations, including cross-hole, seismic CPT, spectral analysis of surface waves (SASW), and suspension logging.  These different methods provide very different resolutions for Vs measurements at different depths.  For example, SASW uses low frequencies to measure deep shear wave velocities, and the resulting measurement is averaged over a large mass of soil, but provides fairly high-resolution measurements near the surface whether other methods are often less accurate.  Spatial resolution with depth suffers as a result, and it is unclear how a point estimate of N should correlate with a vertically averaged estimate of Vs.  Cross-hole methods and suspension logging methods use higher frequency waves that average the properties of a much smaller mass of soil, and therefore provide a higher resolution point estimate similar in spatial scale to an SPT blow count.

The most common functional form for the relations proposed in the literature is Vs=A∙NB, where the constants A and B are determined by statistical regression of a data set.  The N-values are typically not corrected for overburden, but sometimes are corrected for hammer energy, rod length, and sampler inside diameter, in which case N would be replaced by N60. The Vs-values are typically not corrected for overburden.

A significant number of correlations have been published on various soil types (Table 1). Imai and Yoshimura (1975) studied the relationship between seismic velocities and some index properties over 192 samples and developed empirical relationships for all soils. Sykora and Stokoe (1983) pointed out that geological age and type of soil are not predictive of Vs while the uncorrected SPT-N value is most important. Sykora and Koester (1988) found strong statistical correlations between dynamic shear resistance and standard penetration resistance in soils. Iyisan (1996) examined the influence of the soil type on SPT-N versus Vs correlation using data collected from an earthquake-prone area in the eastern part of Turkey. The results showed that, except for gravels, the correlation equations developed for all soils, sand and clay yield approximately similar Vs values. Jafari et al (2002) presented a detailed historical review on the statistical correlation between SPT-N versus Vs. Hasancebi and Ulusay (2006) studied similar statistical correlations using 97 data pairs collected from an area in the north-western part of Turkey and developed empirical relationships for sands, clays, and for all soils irrespective of soil type. Ulugergerli and Uyanık (2007) investigated statistical correlations using 327 samples collected from different areas of Turkey and defined the empirical relationship as upper and lower bounds instead of a single average curve for estimating seismic velocities and relative density.

Table 1. Some existing correlations between Vs and SPT-N

Significant differences exist among the various published relations, which is likely partially caused by differences in geology, but also by errors in measurements of N and Vs.  Resolving the differences among published relationships is beyond the scope of this thesis.

In this thesis, particular attention is given to the functional form used to relate Vs to N60.  Nearly every relation represents Vs directly as a function of N without consideration for vertical effective stress, including the very early relations and more recent ones [e.g., Dikmen (2009) and Hasancebi and Ulusay (2007)].  One exception is Andrus et al. (2004), who related Vs1 to (N1)60 (i.e. overburden corrections are applied to Vs and N60 before regression).  Sykora and Stokoe (1983) evaluated Vs as a function of (N1)60 and found poor correlation and suggested correlating with N60 instead.  Hence, of the 26 relations summarized in Table 1, 25 of them utilize the same functional form and neglect the influence of overburden.

## Influence of Overburden Stress

Vs and N are known to normalize differently with overburden stress, so it is surprising that most correlations relate Vs to N instead of Vs1 to (N1)60.  Equations 1 and 2 are common overburden correction equations for N60 and Vs.

|  | (1) |
| --- | --- |
|  | (2) |

Differences in the manner in which Vs and N60 normalize with v’ would be expected to introduce bias into relations directly between Vs and N60.  For example, consider three different profiles of uniform clean sand with  = 20 kN/m3, (N1)60= 20 and a deep water table as shown in Fig. 2.  Based on the relation by Andrus et al. (2004), Vs1 = 87.8(N1)600.253 = 187 m/s.  Assuming n=0.5 and m=0.25, profiles of N60 and Vs

are plotted in Fig. 2.  To demonstrate bias introduced in the Vs-N relation, the Vs values are also plotted by assuming the Andrus et al. relation directly relates Vs with N60 [rather than Vs1 with (N1)60].  The two methods produce identical estimates of Vs at when v’ = Pa (about 5m depth), but diverge significantly shallower and deeper in the profile.  The only rational method for eliminating the bias is to correlate Vs1 with (N1)60 while utilizing the correct overburden correction factors.

Figure 2. Influence of overburden on Vs values associated with constant N60 but different (N1)60

The exponents n and m in Eqs. 1 and 2 are often taken as 0.5 and 0.25, respectively, but these are empirical constants that exhibit variation.  For example, the exponent m is known from laboratory studies to depend on plasticity index and varies between 0.25 for clean sands and 0.5 for cohesive soil (Yamada et al. 2008).  Furthermore, the cementation that naturally occurs in older sands is known to affect small-strain behavior (i.e. Vs) more than large-strain behavior (i.e. N60).  For example, DeJong et al. (2006) tested loose sand specimens cemented using gypsum and calcite precipitated by bacteria and found that Vs increased by as much as a factor of 4 due to cementation using bender element measurements in triaxial compression specimens.  When the specimens were tested in undrained triaxial compression, the cemented specimens were stiffer initially but converged with the uncemented shear strength.  The standard penetration test induces extremely large strains in the soil in the immediate vicinity of the sampler; hence it would be expected to obscure the influence of age-induced cementation.  Therefore, it is reasonable to anticipate that the exponent m may increase with age, and age increases with depth and v’.  Recent studies have explored the influence of age on SPT-based liquefaction resistance since cementation increases liquefaction resistance more than N60 (e.g., Hayati and Andrus 2009).  Since n and m depend on soil type, cementation, age, and other factors, it is difficult to accurately correct N60 and Vs to obtain (N1)60 and Vs1 to produce a relation that is unbiased with respect to overburden.  On the other hand, a v’ term could be included in the regression of Vs with N60 to inherently define the relative scaling of n and m for a particular data set.  This is the approach adopted in this paper, as explained in chapter 4.

# Development of Data Set

This study utilized data from a set of boreholes at Caltrans bridge sites where SPT N-values and downhole suspension logs were obtained.  A total of 21 bridges and 79 boring logs were identified where N60 and Vs measurements were available from the same borehole (Table 2).  Locations of the bridges are shown in Fig. 3.

Table 2. Bridge name, number, location, number of borings, and surface geology epoch for sites included in this study.

Figure 3. Map of sites where boring logs and suspension logs were measured for this study.

Boring logs were contained in as-built drawings provided by Tom Shantz of Caltrans in tif image format, and were digitized by recording blow count and soil type for each SPT measurement, and the site stratigraphy was also digitized based on the site geologist’s or engineer’s interpretation.  Corrections to the stratigraphy were often made so that transitions in the Vs profile better corresponded to interpreted layer boundaries.  Elevation of the top of the bore hole, ground water elevation, date, GPS coordinates, type of boring, size of borehole, size of sampler, hammer type and sampler type were recorded for each boring log.  A few boreholes had N-values recorded as “P” or “push” when sampler sank under the weight of the rod were excluded from the data set for regression. Some blow counts were also terminated before the sampler was driven the full 18”, and were recorded, for example, as 50/4”.  Such blow counts were excluded from the data set utilized in the regression.

Shear- and p-wave velocity logs were provided by Tom Shantz as Excel files, and were recorded using the downhole suspension logging method explained by Owen (1996).  In this method, a probe is lowered down the open fluid-filled borehole and the source at the tip of the probe excites a wave that propagates through the boring fluid into the soil and is recorded by two receivers at 1m spacing attached to the probe above the source.  The data were subsequently evaluated for quality as explained by Owen (1996), and poor quality data for which the recorded traces were difficult to evaluate were eliminated from the data set.

From the combination of boring logs and suspension logs, a total of 918 N60 values were available where Vs values were recorded at the same depths.  Fig. 4 shows the distribution of available data by soil type, which was determined by visual manual classification in the boring logs.  For some boreholes Vs values were not recorded at shallow depths where N-values were available, and Vs values often were recorded deep in the profile where N60 was not recorded.  Only the combinations where N-values and Vs-values were recorded at the same depth were included in the 918 data points.  All of the data were collected between 1993 and 2001.

Figure 4. Distribution of data points by soil type.

The standard SPT sampler was used for all of the borings and the hammer type was either a safety hammer with an estimated efficiency of 60%, or an automatic hammer with an estimated efficiency of 82% based on previous Caltrans research (Tom Shantz, personal communication).  A rod length correction factor was applied based on the information in Table 2.  A liner correction factor of 1.0 was applied for samplers with liners and 1.2 for samplers without liners, which is in the middle of the range suggested by Youd et al. (2001).   Some information was not included in every boring log.  For example, ground water elevation was sometimes not recorded, in which case the p-wave velocity profile provided in the downhole suspension logs was used to identify the approximate elevation of the ground water table.  Typically an abrupt transition from p-wave velocity lower than 500m/s to 1500m/s or higher was apparent in the boring logs and ground water elevation could be estimated with approximately 1m resolution. Whether the sampler was driven with liners was also not always available, in which case a liner correction of 1.0 was applied. Unit weights were reported only for some samples in the boring logs, and the average values for each soil type (Table 4) were assigned to samples for which unit weight was not reported.

Table 3. Rod length correction factors  (Source: Youd et al., 2001).

Table 4. Unit weights.

Example data from the Noyo River Bridge is shown in Fig. 5. The first two graphs show the Vs profile and N60 profile at the site. The Vs profiles were typically recorded at 0.5m intervals, whereas the N60 values were recorded at much coarser sampling intervals typically 1.5m or larger.  A number of possible approaches were considered for selecting an appropriate Vs value to associate with each N60 value for statistical regression.

Figure 5. Example of data processing methodology to obtain weighted average  value for statistical regression.

The first possibility considered was to select the Vs value at the elevation that is nearest to the elevation where the N60 value was recorded.  This approach was dismissed because high-frequency spatial variations in the Vs profiles could introduce errors in the regression.  SPT N-values are not true point estimates, but rather average out soil properties over a finite region, and it is therefore important to obtain a Vs estimate that exhibits similar averaging.

The approach adopted in this study utilized a weighted average of the Vs profile with the weighting values inversely proportional to the difference in elevation between the N60 measurement and the Vs measurements.  The weights were computed from a truncated normal distribution centered at the N60 elevation with a standard deviation of 1m.  Layer corrections were not applied to the recorded blow counts.  The probability density function was truncated at layer boundaries (i.e. weights were set to zero outside of the stratum containing the N60 value).  The weighted average shear wave velocity, , was computed using Eq. 3.  Fig. 5 shows the weight functions and resulting  values for three different N60 values in the boring log.  Point 1 shows a boring log near the center of a stratum, where the weighting function is not significantly truncated at layer boundaries.  Point 2 shows an N60 value near the bottom of a stratum that is truncated in the sand layer, and does not contain any influence of the underlying silt layer.  Point 3 shows an N60 value near the top of a dense sand layer that is truncated so that the upper looser sand layer does not provide influence.  Fig. 5 also shows an N60 value in the upper gravel layer, which lies above the elevation where Vs measurements commenced.  This N60 value is therefore not associated with a  value and was not included in the regression.  Furthermore, the N60 values terminate when the underlying greywacke rock formation is reached, though Vs values continue into this formation.  are therefore also not available in the greywacke formation and this layer is not included in the statistical regression.

|  | (3) |
| --- | --- |

# Statistical Regression

## Regression Analysis

The form of statistical regression utilized in this study expresses  in terms of N60, v’, and regression constants, where  is in m/s and v’ is in kPa.  The form of the regression equation differs from past studies by including v’, as shown in Eq. 4.

|  | (4) |
| --- | --- |

Re-arranging and expressing in terms of natural logs, the functional form in Eq. 5 is obtained, where 0 = lnA, 1 = B, 2 = n∙B-m, and  is a random error term that is normally distributed with zero mean.  Overburden correction was not directly applied to N60 and  [i.e. Vs1 and (N1)60 were not computed prior to regression], but rather the 2 parameter provides a measure of the relative overburden scaling between  and N60 that minimizes residuals with respect to v’ simultaneously with N60.

|  | (5) |
| --- | --- |

The -constants were solved using ordinary least squares regression according to Eq. 6, where a single underline indicates a vector and a double underline indicates a matrix.

|  | (6) |
| --- | --- |

Regression was performed for sand, silt and clay soil types.  The number of data points for gravel was deemed insufficient for regression.

Results of the regression parameters are contained in Table 5, and plotted in Fig. 6 as  versus N60 and  versus Pa/v’. Trend lines are plotted through the data points corresponding to various Pa/v’ values for  versus N60 and for various N60 values for  versus Pa/v’.  Multiple trend lines are required since the regression includes both N60 and Pa/v’, and the trend lines are useful for identifying the relative influence of N60 and Pa/v’ for the regression of each soil type.

Table 5. Regression constants and standard deviation of error term.

For sand, Pa/v’ exerts significant influence on .  For example, considering sand with N60=30, the regression equation returns median values of  = 323, 274, and 233 m/s for Pa/v’ = 0.25, 0.5, and 1.0, respectively.  This trend is consistent with the example problem in Fig. 2, where for a given N60 value, soil with higher v’ (and lower Pa/v’) exhibits higher Vs.  On the other hand, for Pa/v’=0.5, the regression equations return median values of  = 245, 274, and 310 m/s for N60 = 10, 30, and 100, respectively.  This indicates that, in the range of common engineering interest,  is more strongly related to overburden stress than to blow count.  This finding is significant since the effect of overburden has not been directly quantified in many past studies, and may help explain the large differences among the numerous published relations.

The influence of Pa/v’ decreases for silt and is the lowest for clay.  For example, considering clay with N60=30, the regression equation returns median values of  = 322, 293, and 267 m/s for Pa/v’ = 0.25, 0.5, and 1.0, respectively.  On the other hand, for Pa/v’=0.5, the regression equations return median values of  = 223, 293, and 395 m/s for N60 = 10, 30, and 100, respectively.  Hence, Vs is more strongly related to blow count than overburden stress in the range of engineering interest for clay, which is opposite to the trend for sand.  However, in all cases Vs was influenced by v’, and neglecting the overburden effect would introduce bias into the results, as demonstrated later.

Figure 6. Vs versus N60 and Pa/v’ for (a) sand, (b) silt, and (c) clay, and lines showing results of regression equations for various N60 and Pa/v’ values.

The 2 values obtained by the statistical regression provide information about the relative overburden scaling for Vs and N60.  Individual values of n and m cannot be solved from the regression, but one can be computed based on the regression parameters 1 and 2 since 2 = n∙1-m. For example, if n=0.5, then m=0.29, 0.38, and 0.26 for sand, silt and clay, respectively.  However, n and m are known to depend on soil type (e.g., Yamada et al. 2008) and contain uncertainty.  Correcting N60 and Vs prior to regression might cause bias with respect to v’, which is eliminated in the regression approach adopted in this study.

## Residuals

Residuals defined as ln() – [0 + 1ln(N60) + 2ln(Pa/v’)] are plotted versus N60 and (Pa/v’) in Fig. 7.  The mean value of the residuals is zero, and there is no trend in the residuals with either N60 or (Pa/v’), which indicates that the regression has removed bias with respect to these input variables.  The standard deviation of the residuals is 0.286, 0.282, 0.290 and 0.306 for sand, silt, clay and all soils respectively. However, the data sample is heteroskedastic, with higher standard deviations at low confining stress.  The larger uncertainty at shallow depths may be caused by measurement uncertainty, though the cause is not known.  In addition to computing the standard deviation of the residuals, the distribution is required for quantifying the error term in Eq. 5.   Normality of the residuals is examined using the quantile-quantile (Q-Q) plots in Fig. 8.  The Q-Q plots represent the sorted residuals (i.e. the quantiles) versus the theoretical residuals that would be anticipated if the error term is normally distributed.

Figure 7. Residuals, defined as natural log of measured Vs values minus natural log of predicted Vs values, versus N60 and Pa/v’ for (a) sand, (b) silt, and (c) clay.

When measured quantiles are plotted against theoretical quantiles, a normally-distributed variable exhibits a linear Q-Q plot with a slope of unity, whereas deviation from normality is manifested by data points that do not lie along the 1:1 line.  Some deviations at the ends of the Q-Q plots are anticipated based on sampling variability since the tails of distributions are often not well-sampled.  The error terms in this case are reasonably approximated by a normal distribution, and deviation from normality is slightly positive at the tails of the distribution, indicating that the tails may contain slightly more probability density than implied by a normal distribution.  However, this may also be due to sampling error, and the data support the conclusion that the error term is normally distributed.

Figure 8. Quantile-quantile plots indicating the extent to which the error term is normally distributed for (a) sand, (b) silt, and (c) clay.

## Influence of Surface Geology

Fig. 9 plots the same data as Fig. 6, but with different symbols corresponding to Holocene, Pleistocene, and Pre-Quaternary geologic epochs for the soil at the surface.  Surface geology mapping was based on a Caltrans-funded study by Keith Knudsen (not yet published) where the surface geologic epoch at all Caltrans bridge sites was obtained.  The data show no significant trend with surface geology.

Figure 9. Vs versus N60 with different symbols indicating different surface geologic epoch for (a) sand, (b) silt, and (c) clay.

In fact, soil with Pleistocene surface geology exhibits slightly lower average N60 and Vs values than soil with Holocene surface geology, though the differences are not statistically significant and are likely related to the fact that the average overburden stress is also smaller for the Pleistocene data points.  The observation of non-dependence on surface geology is surprising since age is known to be associated with cementation and higher strength and stiffness, but the observation is consistent with a number of previous studies (e.g., Sykora and Stokoe 1983) that also found that surface geology did not affect the correlation.  One possible explanation for the non-dependence is that the Vs-N60 pairs came from a range of depths, and sites with Holocene surface geology may be underlain by Pleistocene deposits.  Hence, many of the data points in Fig. 9 associated with a Holocene symbol may have actually come from a Pleistocene or Pre-Quaternary geologic unit, particularly those with high v’ since these samples were deeper in the profile.

## Effect of Omitting v’ Term

To demonstrate the bias caused by omitting the overburden term from regression, the regression analysis has been carried out directly between N60 and  for all soils, sand, silt and clay. The following formulae were obtained by using the existing dataset:

|  | for all soils | (7) |
| --- | --- | --- |
|  | for sands | (8) |
|  | for silts | (9) |
|  | for clays | (10) |

The residuals of the data for sand are plotted versus N60 and versus Pa/v’ in Fig. 10. As expected, there is no bias with respect to N60.  However, there is significant bias with respect to the overburden term. Residuals are positive at high overburden stress (i.e. at low Pa/v’), indicating Vs is underestimated, and negative at low overburden stress (i.e. at high Pa/v’).  This evidence suggests that similar bias is present in published relations that directly correlate Vs with N, which may help explain why the published relations differ so significantly from each other.

Figure 10: Residuals versus N60 and Pa/σv' for regression for all soils without overburden term.

## Comparison with Published Relations

Some of correlations from Table 1 are plotted against the data in Fig 11. Some of the correlations fit the data points reasonably well, though there is tremendous difference in the range of Vs values predicted for a given N value.  It is unclear how much of these deviations are caused by natural variability in soil deposits, how much is caused by errors in measurements of N and Vs, and how much is caused by exclusion of overburden correction in the existing relations.  For example Kanai (1966) may have utilized data recorded primarily at shallow depths, which could largely explain why their relation is lower than the others.  Future efforts should aim to reduce the variability in these relations by utilizing only high-quality measurements of N and Vs, and properly incorporating the influence of overburden.  This effort would involve re-interpretation of the data available in published relations, which is beyond the scope of this thesis.

Figure 11: Existing correlations from literature over the current data set

# Conclusions

In this study, shear wave velocity, Vs, was related to N60 and Pa/v’.  A total of 21 bridges and 79 boring logs were identified where N60 and Vs measurements were available from the same borehole for the study from Caltrans. Only the combinations where N-values and Vs-values were recorded at the same depth were included in the 918 data points for the regression analysis. Published relations typically neglect the influence of overburden, which was shown in this study to be an important contributor when predicting Vs.  In fact, Vs is correlated more closely with Pa/v’ than with N60 for sand.  This implies that if you had to predict Vs using either N60 or Pa/v’ (but not both), Pa/v’ would on average provide a more accurate estimate.  Of course, knowing both N60 and v’ would provide a better estimate, but this simple finding demonstrates a significant deficiency in most published relations, which neglect overburden effects.  For silt and clay, Vs correlated more closely with N60 than Pa/v’, but overburden nevertheless exerted an important influence.

N60 and Pa/v’ are not exceptionally efficient predictors of Vs, and the resulting uncertainty is significant as demonstrated by the large scatter of the data in Fig. 6.  The standard deviation of the residuals was about 0.3, which implies a coefficient of variation of about 30%.  This large uncertainty is not surprising since Vs is a small-strain property, while N is a large-strain property.  If the outcome of a problem depends significantly on Vs, correlations with blow count should not be relied upon.  Rather, geophysical measurements should be made since any method for measuring Vs would be anticipated to provide a more accurate estimate than the proposed relations.

A likely application of the correlations presented in this work is the calculation of the thirty-meter shear wave velocity, Vs30, which is defined as 30m divided by the travel time of a vertically propagating shear wave in the upper 30m.  Vs30 is a required input for the Next Generation Attenuation models [Boore and Atkinson (2008), Campbell and Bozorgnia (2008), Abrahamson and Silva (2008), Chiou and Youngs (2008)] and is therefore needed to quantify seismic hazard.  Geotechnical site investigations at many older sites contain boring logs, but no geophysical measurements.  Obtaining a rough estimate of Vs30 based on the recorded boring logs could therefore be useful for assessing seismic hazard at sites with that lack geophysical measurements, and for identifying whether geophysical measurements are necessary to further refine the estimate of Vs30.  Though beyond the scope of this thesis, a useful study would compare Vs30 values obtained directly by the suspension logs with those obtained by correlation with N60 and Pa/v’.  Regarding the manner in which uncertainty in the point estimate of Vs propagates through to calculation of Vs30, a particularly important concept is division of the error term into inter-boring and intra-boring components.  Inter-boring error is the scatter of the data points about a trend for that particular boring, while intra-boring error identifies whether estimated Vs values for a given boring are systematically under- or over-estimated using the proposed relations.  If intra-boring error terms are small, then the variability in point estimates of Vs would be canceled out since an approximately equal number of over-predictions and under-predictions would occur.  In this case, Vs30 estimates would be reasonably accurate using the proposed relations.  On the other hand, if intra-boring errors are significant, then Vs30 would be systematically under-predicted at high Vs30 sites and systematically over-predicted at low Vs30 sites.  By inspection of the plots in the Appendix, intra-boring error terms are significant for this data set, and more work is needed to quantify uncertainty in Vs30.

The proposed relations are not an accurate substitute for geophysical measurements, and uncertainty in the predictions should be considered when using the relations.  The primary purpose of the relations is to aid decisions regarding whether geophysical measurements are warranted for a particular problem and site.

# Appendix

This appendix includes plots of vertical effective stress, SPT blow count (N60), measured shear wave velocity (Vs) and predicted shear wave velocity versus elevation for all boring logs utilized in this study.

Figure 12. (a) Bridge no. 10-0298, boring no. 98-4 (abut.4), (b) Bridge no. 10-0298, boring no. 96-2, (c) Bridge no. 10-0298, boring no. 96-3.

Figure 13. (a) Bridge no. 10-0298, boring no. 98-10 (abut 1) (b) Bridge no. 28-0253R, boring no. 94B1R (Pier 9) (c) Bridge no. 28-0153R, boring no. 96-5 (Pier 8)

Figure 14. (a) Bridge no. 28-1053R, boring no. 95B13R (Pier 7) (b) Bridge no. 28-1053R, boring no. 96-4 (Pier 7) (c)  Bridge no. 28-1053R, boring no. 95-12 (Pier 5)

Figure 15. (a) Bridge no. 28-0352L, boring no.  96B-29 (b)  Bridge no. 28-0352L, boring no. 95-2 (Pier 3) (c)  Bridge no. 28-0352L, boring no. 95-1 (Pier 4)

Figure 16. (a) Bridge no. 28-0352L, boring no. 96B-37 (b) Bridge no. 28-0100, boring no. 96-2 (Piers 10 and 11) (c) Bridge no. 28-0100, boring no. 96-5 (Piers 31/32)

Figure 17. (a) Bridge no. 28-0100, boring no.  96-7 (Pier 8) (b) Bridge no. 28-0100, 95-7 (Pier 21) (c) Bridge no. 28-0100, boring no. 95B4R (Pier 25)

Figure 18. (a) Bridge no. 28-0100, boring no.  95B5R (Pier 35) (b) Bridge no. 28-0100, boring no. 95B2R (Pier 32/33) (c) Bridge no. 28-0100, boring no. 95B3R/95B9R (Pier 34)

Figure 19. (a) Bridge no. 28-0100, boring no. 95-10 (Pier 47) (b)  Bridge no. 28-0100, boring no. 95-11 (Pier 48) (c)  Bridge no. 28-0100, boring no. 95B1R (Pier 58)

Figure 20. (a) Bridge no. 33-0025, boring no. B6 (Pier E19) (b) Bridge no. 33-0025, boring no. B-7 (Pier E10) (c) Bridge no. 34-0003, boring no. 95-14 (Pier W6)

Figure 21. (a) Bridge no. 34-0003, boring no. 95-12 (Pier W4) (b) Bridge no. 34-0003, boring no. 95-11 (Pier W3) (c) Bridge no. 34-0003, boring no. 95-10 (Pier W2)

Figure 22. (a) Bridge no. 34-0003, boring no. 95-5 (Pier A) (b) Bridge no. 34-0003, boring no. 95-4 (c) Bridge no. 34-0003, boring no. 95-6

Figure 23. (a) Bridge no. 34-0004, boring no. B95-2 (b) Bridge no. 34-0004, boring no. B95-3 (c) Bridge no. 34-0077, boring no. 01-B2

Figure 24. (a) Bridge no. 34-0077, boring no. 01-05 (b) Bridge no. 34-0077, boring no. 01-08 (c) Bridge no. 34-0077, boring no. 01-11

Figure 25. (a) Bridge no. 37-0853, boring no.98-1 (Pier 4) (b) Bridge no. 38-0583, boring no. 98-4 (Bent 7) (c) Bridge no. 49-0014L, boring no.98-1 (Abut 1)

Figure 26. (a) Bridge no. 51-0139, boring no. 98-1 (Abut 1) (b) Bridge no. 52-0443, boring no. 99-1 (c) Bridge no. 53-1471, boring no. 95B5R

Figure 27. (a) Bridge no. 53-1471, boring no. 95B4R (b) Bridge no. 53-1471, boring no. 95B1R (c) Bridge no. 53-1471, boring no. 95B2R

Figure 28. (a) Bridge no. 53-1471, boring no. 95B3R (b) Bridge no. 53-2272, boring no. B-1 (c) Bridge no. 53-2790R, boring no. B-6

Figure 29. (a) Bridge no. 53-2794R, boring no. B-1 (b) Bridge no. 53-2795F, boring no. 94-21 (c) Bridge no. 53-2796F, boring no. 94-30

Figure 30. (a) Bridge no. 54-1110R, boring no. 98-1 (Abut 1) (b) Bridge no. 54-1110R, boring no. 98-6 (Abut 8) (c) Bridge no. 57-0857, boring no. 96-52 (Bents R48 and 49)

Figure 31. (a) Bridge no. 57-0857, boring no. 96-17 (Abut S48) (b) Bridge no. 57-0857, boring no. 95-2 (Pier 33) (c) Bridge no. 57-0857, boring no. 96-16 (Bent 41F,R)

Figure 32. (a) Bridge no. 57-0857, boring no. 96-29 (b) Bridge no. 57-0857, boring no. 96-53R (c) Bridge no. 57-0857, boring no. 96-66

Figure 33. (a) Bridge no. 57-0857, boring no. 96-65 (b) Bridge no. 57-0857, boring no. 96-35 (Toll Plaza North West) (c) Bridge no. 57-0857, boring no. 96-34 (Toll Plaza South East)

Figure 34. (a) Bridge no. 57-0857, boring no. 96-21 (b) Bridge no. 57-0857, boring no. 96-28 (c) Bridge no. 57-0857, boring no. 96-60

Figure 35. (a) Bridge no. 57-0857, boring no. 96-68R (b) Bridge no. 57-0857, boring no. 96-56 (c) Bridge no. 57-0857, boring no. 96-67

Figure 36. (a) Bridge no. 57-0857, boring no. 96-54 (b) Bridge no. 57-0857, boring no. 96-55 (c) Bridge no. 57-0857, boring no. 96-59

Figure 37. (a) Bridge no. 57-0857, boring no. 96-58 (b) Bridge no. 57-0857, boring no. 96-57 (c) Bridge no. 57-0857, boring no. 96-64

Figure 38. (a) Bridge no. 58-0335RL, boring no. B5-01

# References

Andrus, R. D., Piratheepan, P., Ellis, B.S., Zhang, J., and Juang, C. H. (2004). “Comparing Liquefaction Evaluation Methods using Penetration-Vs relationships”, Soil Dynamics and Earthquake Engineering, 24, 713-721.

Athanasopoulos, G. A. (1995). “Empirical correlations Vs-NSPT for soils of Greece: a comparative study of reliability,” Proc. 7th Int. Conf. on Soil Dynamics and Earthquake Engineering (Chania, Crete) ed A S Cakmak (Southampton: Computational Mechanics), 19–36.

Barrow, B. L. and Stokoe, K. E. II (1983). “Field investigation of liquefaction sites in Northern California,” Geotechnical Engineering Thesis, GT 83-1, Civil Engineering Department, University of Texas at Austin, 12.

Boore, D. M., and Atkinson, G. M. (2008). “Ground-motion prediction equations for the average horizontal component of PGA, PGV, and 5%-damped PSA at spectral periods between 0.01 s and 10.0 s,” Earthquake Spectra, 24, 99–138.

Campbell, K.W. and Duke, C.M. (1976). “Correlations among seismic velocity, depth and geology in the Los Angeles area,” Report ENG-7662, University of California at Los Angeles School of Engineering and Applied Science.

Campbell, K. W., and Bozorgnia, Y. (2008). “Campbell-Bozorgnia NGA horizontal ground motion model for PGA, PGV, PGD and 5% damped linear elastic response spectra,” Earthquake Spectra, 24, 139–171.

Chiou, B., and Youngs, R. R. (2008). “An NGA model for the average horizontal component of peak ground motion and response spectra,” Earthquake Spectra, 24, 173–215.

Choi, Y. and Stewart, J.P. (2005). “Nonlinear site amplification as function of 30 m shear wave velocity,” Earthquake Spectra, 21 (1), 1-30.

Dejong, J. T., Fritzges, M. B., and Nusslein, K. (2006). “Microbially Induced Cementaion to Control Sand Response to Undrained Shear”, Journal of Geotechnical and Geoenvironmental Engineering, Vol. 132, No.11, 1381-1392.

Dikmen, U. (2009). “Statistical correlations of shear wave velocity and penetration resistance for soils,” Journal of Geophysics and Engineering, 6, 61-72.

Fujiwara, T. (1972). “Estimation of ground movements in actual destructive earthquakes,” Proc. 4th European Symp. Earthquake Engineering (London), 125–32.

Hasancebi, N. and Ulusay, R. (2006). “Empirical correlations between shear wave velocity and penetration resistance for ground shaking assessments,” Bull Eng Geol Environ, 66, 203-213.

Hayati, H., and Andrus, R. D. (2009). “Updated Liquefaction Resistance Correction Factors for Aged Sands”, Journal of Geotechnical and Geoenvironmental Engineering, Vol. 135, No.11, 1683-1692.

Imai, T. (1977). “P-and S-wave velocities of the ground in Japan,” Proc.9th Int. Conf. on Soil Mechanics and Foundation Engineering, vol 2,  127–32.

Imai, T. and Tonouchi, K. (1982). “Correlation of N-value with S-wave velocity and shear modulus,” Proc. 2nd European Symp. Of Penetration Testing (Amsterdam), 57–72.

Imai, T. and Yoshimura, Y. (1975). “The relation of mechanical properties of soils to P and S-wave velocities for ground in Japan,” Technical Note, OYO Corporation.

Iyisan, R. (1996). “Correlations between shear wave velocity and in-situ penetration test results,” Tech. J. Chamber Civil Eng. Turkey, 7, 1187–99 (in Turkish).

Jafari, M. K., Shafiee, A. and Ramzkhah, A. (2002). “Dynamic properties of the fine grained soils in south of Tehran,” J. Seismol. Earthq. Eng., 4, 25–35.

Jafari, M. K., Asghari, A. and Rahmani, I. (1997). “Empirical correlation between shear wave velocity (Vs) and SPT-N value for south of Tehran soils,” Proc. 4th Int. Conf. on Civil Engineering (Tehran, Iran) (in Persian).

Jinan, Z. (1987). “Correlation between seismic wave velocity and the number of blow of SPT and depth,” Chin. J. Geotech. Eng. (ASCE), 92–100 (selected papers).

Kanai, K. (1966). “Conf. on Cone Penetrometer The Ministry of Public Works and Settlement (Ankara, Turkey),”  (presented by Y Sakai 1968).

Kiku, H., Yoshida, N., Yasuda, S., Irisawa, T., Nakazawa, H., Shimizu, Y., Ansal, A. and Erkan, A. (2001). “In-situ penetration tests and soil profiling in Adapazari, Turkey,” Proc. ICSMGE/TC4 Satellite Conf. on Lessons Learned from Recent Strong Earthquakes, 259–65.

Lee, S. H. (1990). “Regression models of shear wave velocities,” J. Chin. Inst. Eng., 13, 519–32.

Ohba, S. and Toriuma, I. (1970). “Dynamic response characteristics of Osaka Plain,” Proc. Ann. Meeting AIJ (in Japanese).

Ohsaki, Y. and Iwasaki, R. (1973). “On dynamic shear moduli and Poisson’s ratio of soil deposits,” Soil Found., 13, 61–73.

Ohta, Y. and Goto, N. (1978). “Empirical shear wave velocity equations in terms of characteristic soil indexes,” Earthq. Eng. Struct. Dyn., 6, 167–87.

Ohta, T., Hara, A., Niwa, M. and Sakano, T. (1972). “Elastic shear moduli as estimated from N-value,” Proc. 7th Ann. Convention of Japan Society of Soil Mechanics and Foundation Engineering, 265–8.

Okamoto, T., Kokusho, T., Yoshida, Y. and Kusuonoki, K. (1989). “Comparison of surface versus subsurface wave source for P–S logging in sand layer,” Proc. 44th Ann. Conf. JSCE, vol 3, 996–7 (in Japanese).

Owen, W. (1996), “Interim Report of PS Velocity for Borelog 96-2, SFOBB East Viaduct Seismic Retrofit,”  Memorandum, State of California.

Pitilakis, K., Raptakis, D., Lontzetidis, K. T., Vassilikou, T. and Jongmans, D. (1999). “Geotechnical and geophysical description of Euro-Seistests, using field and laboratory tests, and moderate strong ground motions,” J. Earthq. Eng., 3, 381–409.

Seed, H. B. and Idriss, I. M. (1970). “Soil Moduli and Damping Factors for Dynamic Response Analyses,” Earthquake Engineering Research Center, University of California, Berkeley, California, Rep. No. EERC-70/10.

Seed, H. B. and Idriss, I. M. (1981). “Evaluation of liquefaction potential sand deposits based on observation of performance in previous earthquakes,” ASCE National Convention (MO), 81–544.

Shibata, T. (1970). “Analysis of liquefaction of saturated sand during cyclic loading,” Disaster Prevention Res. Inst. Bull., 13, 563–70.

Sisman, H. (1995). “The relation between seismic wave velocities and SPT, pressuremeter tests,” MSc Thesis, Ankara University (in Turkish).

Sykora, D. E. and Stokoe, K. H. (1983). “Correlations of in-situ measurements in sands of shear wave velocity,” Soil Dyn. Earthq. Eng., 20, 125–36.

Sykora, D. W. and Koester, P. J. (1988). “Correlations between dynamic shear resistance and standard penetration resistance in soils,” Earthq. Eng. Soil Dyn., 2, 389–404.

Tamura, I. and Yamazaki, F. (2002). “Estimation of S-wave velocity based on geological survey data for K-NET and Yokohama seismometer network,” J. Struct. Mech. Earthq. Eng., I, 237–48 (in Japanese).

Ulugergerli, U. E. and Uyanik, O. (2007). “Statistical correlations between seismic wave velocities and SPT blow counts and the relative density of soils,” J. Test. Eval., 35, 1–5.

Vucetic, M. and Dobry, R. (1991). "Effect of Soil Plasticity on Cyclic Response," ASCE Journal of Geotechnical Engineering, Vol. 117, No. 1, 89-107.

Yamada, S., Hyodo, M., Orense, R. P., and Dinesh, S. V. (2008). “Initial Shear Modulus of Remolded Sand-Clay Mixtures”, Journal of Geotechnical and Geoenvironmental Engineering, Vol. 134, No.7, 960-971.

Youd, T.L., Idriss, I.M., Andrus, R.D., Arango, I., Castro, G., Christian, J.T., Dobry, R., Finn, W.D.L., Harder Jr., L.F., Hynes, M.E., Ishihara, K., Koester, J.P., Liao, S.S.C., Marcuson III, W.F., Martin, G.R., Mitchell, J.K., Moriwaki, Y., Power, M.S., Robertson, P.K., Seed, R.B., and Stokoe II, K.H. (2001). “Liquefaction Resistance of Soils: Summary Report from the 1996 NCEER and 1998 NCEER/NSF Workshops on Evaluation of Liquefaction Resistance of Soils.” J. Geotech. Geoenvrion. Eng. 127(10), 817-833.